Ministry of Oil               	South Gas Company                                                                                                                                                                                                        (      (State Company

SGC-C178-2012

SUBJECT :TECHNICAL I OFFER FOR ( NON SPARKING BOLTING TOOLS (HYDRAULIC TYPE )WITH  ALL CONNECTION)

1-Complete hydraulic Nut Splitters with hand pump :

-Qt.:"2"two Set

-Capacity :5-90 Ton

-Bolt range :M12-M48

-Maximum Operating Pressure:700 bar

-Length of  hose :6 m

| M39-M48 | M33-M39 | M27-M33 | M22-M27 | M16-M22 | M12-M16 | Bole range (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| 60-75 | 50-60 | 41-50 | 32-41 | 24-32 | 19-24 | Hexagon Nut range(mm) |

2-Complete hydraulic flange Spreader with hand pump :

-Qt.:"2"two Set

-Capacity : 5 – 10 Ton

-Maximum Operating  Pressure ; 700 bar

-Length of hose :6m

| Capacity \(ton) | Standard wedge (mm) | Stud size (mm) | Maximum flange thickness (mm) |
| --- | --- | --- | --- |
| 5 | 3-28 | 19-28 | 2x57 |
| 10 | 3-28 | 31-41 | 2 x 92 |

3-Complete hydraulic wedge spreaders with hand pump & safety block :

-Qt.:"2" two Set

-Tip clearance :6mm

-Maximum spread :6mm/81mm .

-Maximum spread force :14 ton

- Maximum Operating pressure :700 bar

-Length of  hose : 6m

4-Flange Alignment tools:

-Qt."1" One Set

-Bolt hole range "17-54" mm

-Maximum lifting force :3 ton

Notes :

1-Origin :West Europe .
2-Training Course for two  technicians in  country of origin (One week ) .

3-Catalogues needed , otherwise ,offers will be not identical .

4-Origin certificates when supplying materials .