REPUBLIC OF IRAQ
[image: image1.png]



MINISTRY OF OIL
SOUTH  GAS COMPANY
STATE  COMPANY
REQ./ SGC – M 207   – 2012
LOW  VOLTAGE  MOTORS
	الكمية المطلوبة

REQ.QTY
	الوحدة

UNIT
	المواصفات

DESCRIPTION
	التسلسل

ITEM

	
	
	LOW  VOLTAGE  MOTORS  FOR  COMPRESSOR  STATIONS  SOUTH  RUMAILA
	

	4
	NOS
	HORIZONTAL  INDUCATION  MOTORS 

THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  110
R.P.M                 :  1475
RATING CONTINOUS ,  CONNECTION  :  STAR / DELTA
MOUNTING : B3  
TYPE :  7 – DS280 MR       OR EQUIVALENT
	1

	4
	NOS
	HORIZONTAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  45
R.P.M                 :  1478
RATING CONTINOUS ,  CONNECTION  :  STAR / DELTA
MOUNTING : B3  
TYPE :  C180177        OR EQUIVALENT
	2

	4
	NOS
	HORIZONTAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  37
R.P.M                 :  2950
RATING CONTINOUS ,  CONNECTION  :  DELTA
MOUNTING : B3  
TYPE :  NVE 200L2       OR EQUIVALENT
	3

	4
	NOS
	HORIZONTAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  15

R.P.M                 :  1450
RATING CONTINOUS ,  CONNECTION  :  DELTA
MOUNTING : B3    
TYPE :  NVPE 160L4         OR EQUIVALENT
	4

	4
	NOS
	HORIZONTAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  11

R.P.M                 :  2890

RATING CONTINOUS ,  CONNECTION  :  DELTA
MOUNTING : B3  
TYPE :  NVPE 160 Ma2   OR EQUIVALENT
	5

	2
	NOS
	HORIZONTAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  7.5
R.P.M                 :  1460

RATING CONTINOUS ,  CONNECTION  :  DELTA
MOUNTING : B3  
TYPE :  160 M       OR EQUIVALENT
	6

	8
	NOS
	VERTICAL  INDUCATION  MOTORS 
THREE  PHASE    EXPLOSION   PROOF  ELECTRIC  INDUCATION  MOTOR  ,  INSULATION  CLASS   F  ,   PROTECTION   IP  55  , 380  V 50  HZ  ,  EEX  d-IIB  T4 ,   AMBIENT  TEMPERATURE  50 C°                   

POWER (KW)   :  0.75
R.P.M                 :  1470
RATING CONTINOUS ,  CONNECTION  :  DELTA
MOUNTING : V1  
TYPE :  eLA  80 b – 4f      OR EQUIVALENT     
	7

	1
	LOT
	TRAINING  FOUR  PERSON  TWO WEEKS  IN  MANUFACTURE WORKSHOP
	

	
	
	 ORIGIN:
 USA , WEST EUROPE  , JAPAN
	


